QuadHopper Reward Function Details Report

Derived directly from train.py

# 1. Reward Overview

The policy reward is the sum of nine implemented terms. Every active term is multiplied by the policy time step dt, so the reward is approximately rate-normalized with respect to the policy update frequency.

Total reward: r = r_survival + r_xy + r_height + r_yaw + r_attitude + r_horizontal_velocity + r_power + r_synergy + r_anti_synergy + r_lazy_ground

Important implementation note: the configured high_power, synergy, and anti_synergy weights are not used by the current compute_reward() equations. The synergy and anti_synergy reward terms are explicitly set to zero.

# 2. Reward Terms and Weights

| Reward term | Weight | Implemented equation | Meaning |
| --- | --- | --- | --- |
| Survival | 5.0 | +5.0 dt | Constant positive reward for remaining in the episode. |
| XY position | 13.0 | +13 exp(-d_xy/0.80) dt | Encourages the robot to remain close to the horizontal origin. |
| Height | 40.0 | +40 exp(-¦z* - z¦/0.20) dt | Strongly encourages the absolute body height to match the fixed target height. |
| Yaw | 4.0 | -4 e_yaw² dt | Penalizes rotation away from the initial yaw. |
| Attitude | 15.0 | -15(q_x² + q_y²) dt | Penalizes roll/pitch-related quaternion components to keep the body upright. |
| Horizontal velocity | 4.0 | -4(v_x² + v_y²) dt | Penalizes lateral/horizontal body motion. |
| Power / effort | 0.03 | -0.03 Σu_i² dt | Penalizes squared normalized motor commands; this is an effort proxy, not physical electrical/mechanical power. |
| Synergy | 0.02 configured | 0 | No effect in the current implementation. |
| Anti-synergy | 0.02 configured | 0 | No effect in the current implementation. |
| Lazy ground | 10.0 | -10 dt if z < 0.3 m; otherwise 0 | Discourages the robot from staying close to the ground. |

# 3. Detailed Mathematical Definition

## 3.1 Survival reward

r_survival = 5.0 dt

A constant positive reward at every policy step. Because the environment currently reports no failure termination, this mainly provides a positive baseline throughout the 30 s episode.

## 3.2 Horizontal-position reward

d_xy = ||e_xy||₂;   r_xy = 13 exp(-d_xy / 0.80) dt

e_xy is the 2-D horizontal position error. In the current environment it is passed as position[:, :2], so the target is the world XY origin. The exponential kernel has a length scale of 0.80 m. At d_xy = 0, this term reaches its maximum rate of 13.

## 3.3 Height reward

e_h = |z_target - z|;   r_height = 40 exp(-e_h / 0.20) dt

This is the largest positive task reward. z_target is fixed at 1.2 m. The exponential length scale is 0.20 m, so the reward decreases rapidly as the robot moves away from the desired height.

## 3.4 Yaw penalty

e_yaw = wrap_to_pi(yaw - yaw_start);   r_yaw = -4 e_yaw² dt

Penalizes squared wrapped yaw deviation from the yaw at episode reset. The error is constrained to the interval approximately [-π, π).

## 3.5 Attitude penalty

r_attitude = -15(q_x² + q_y²) dt

Uses the x and y quaternion components as the attitude-error signal. It is intended to suppress roll and pitch and keep the body approximately upright.

## 3.6 Horizontal-velocity penalty

r_vxy = -4(v_x² + v_y²) dt

Penalizes horizontal body velocity. This complements the XY-position reward by discouraging drift and oscillatory lateral motion.

## 3.7 Motor-effort penalty

P_proxy = Σ(i=1..4) u_i²;   r_power = -0.03 P_proxy dt

u_i are the four clipped motor commands in [-1, 1]. Despite the variable name 'power', this is not τ·qdot or measured electrical power; it is the squared action magnitude.

## 3.8 Synergy and anti-synergy

r_synergy = 0;   r_anti_synergy = 0

Weights of 0.02 are declared for both terms, but the current reward implementation assigns zeros. Therefore these terms do not affect training.

## 3.9 Lazy-ground penalty

r_lazy = -10 dt, if z < 0.3 m; otherwise 0

A threshold penalty that discourages low-height behavior. The threshold is 0.3 m, slightly above the configured initial contact height of about 0.27335 m.

# 4. Definition of Reward Variables

| Variable | Definition | Unit | Implementation detail |
| --- | --- | --- | --- |
| dt | Policy time step | s | step_dt. Simulation dt = 0.005 s and decimation = 2, implying a 0.01 s / 100 Hz policy step. |
| e_xy | Horizontal position error vector | m | Current XY position relative to the world origin: [x, y]. |
| d_xy | Horizontal distance error | m | Euclidean norm ¦¦e_xy¦¦₂. |
| z | Current body/robot height | m | position[:, 2]. |
| z_target | Target absolute height | m | Fixed at 1.2 m. |
| e_h | Absolute height error | m | ¦z_target - z¦. |
| yaw | Current yaw angle | rad | Current heading state. |
| yaw_start | Episode-start yaw | rad | Reset to 0 in the current environment. |
| e_yaw | Wrapped yaw error | rad | wrap_to_pi(yaw - yaw_start). |
| q_x, q_y | Quaternion x/y components | - | Used as the roll/pitch attitude penalty signal. |
| v_x, v_y | Horizontal body velocity components | m/s | First two components of velocity. |
| u_i | Normalized motor command | - | Four actions clipped to [-1, 1]. |
| P_proxy | Motor-effort proxy | - | Sum of squared commands Σu_i²; not physical power. |
| spring_position | Spring position | implementation state | Provided to RewardInputs but not used by any active reward term. |
| spring_velocity | Spring velocity | implementation state | Provided to RewardInputs but not used by any active reward term. |

# 5. Interpretation of Relative Reward Strength

At zero tracking error, the positive reward rates before multiplication by dt are 5 (survival), 13 (XY), and 40 (height), for a maximum positive rate of 58 before penalties. Thus height regulation is the dominant explicitly weighted objective, followed by XY centering. Attitude stabilization has a relatively strong quadratic penalty coefficient (15), while yaw and horizontal velocity each use coefficient 4. The action-effort coefficient is comparatively small (0.03), although its actual influence depends on the magnitude and persistence of the four motor commands.

With the configured 100 Hz policy rate (dt ≈ 0.01 s), the per-step maxima at perfect tracking are approximately: survival = 0.05, XY = 0.13, height = 0.40, giving 0.58 per step before penalties.

# 6. Important Implementation Observations

The target height is an absolute world-Z target fixed at 1.2 m.

The XY target is implicitly the world origin because xy_error is set directly to position[:, :2].

The 'power' term is a normalized action-effort penalty, not a physically calibrated power or energy term.

high_power = 0.50 is declared in RewardWeights but is not referenced in compute_reward().

synergy = 0.02 and anti_synergy = 0.02 are declared, but both implemented reward terms are zero.

spring_position and spring_velocity are supplied to RewardInputs but do not contribute to the current reward.

The environment's done function currently sets failure=False for every environment; episodes end by timeout rather than a reward-related failure condition.

# 7. Compact Reward Equation

r = dt [ 5 + 13 exp(-||e_xy||₂/0.80) + 40 exp(-|z_target-z|/0.20) - 4 e_yaw² - 15(q_x²+q_y²) - 4(v_x²+v_y²) - 0.03 Σu_i² - 10 I(z<0.3) ]

where I(·) is an indicator function. The synergy and anti-synergy terms are omitted from this compact equation because their current implemented values are identically zero.

# 8. Source Scope

This report describes the reward exactly as implemented in the supplied train.py. It does not infer additional reward terms from external QuadHopper or Isaac Lab implementations.